# Configurable Serial-to-Parallel Receiver Using Verilog HDL

## System Specs

The system is a serial-to-parallel receiver that receives serial data bit-by-bit and converts it into parallel data bytes.

The system supports:
- Start/stop bit detection
- Fixed baud-rate timing (1200 bps)
- Even parity checking
- Error detection
- Data-valid indication
- Configurable data size

In your test, use 100MHz clock

## Top Module

Module Name: serial_to_parallel_receiver

## Inputs

input clk
input rst_n
input serial_in
input enable
input parity_enable
input [1:0] data_size

## Outputs

output reg [7:0] parallel_data
output reg data_valid
output reg parity_error
output reg framing_error

## Data Size Configuration

00 -> 4 bits
01 -> 5 bits
10 -> 6 bits
11 -> 7 bits

Unused upper bits of parallel_data should be filled with zeros.

## Packet Format

START | DATA | PARITY | STOP

Start Bit:
- Logic 0

Data Bits:
- LSB first

Parity:
- Even parity only

Stop Bit:
- Logic 1

## Receiver FSM States

- IDLE
- START
- RECEIVE
- PARITY
- STOP
- DONE
- ERROR

## Shift Register Requirements

- LSB-first shifting
- Variable shift length
- Synchronous operation

## Counters

- Bit counter
- Clock counter for baud timing

## Timing Requirements

The receiver uses a fixed baud rate of 1200 bps. ( Please Use 100 MHz clock in your test)

## Error Detection

The receiver must detect:
- Even parity errors
- Framing errors

## Reset Behavior

On reset:
- FSM returns to IDLE
- Counters reset
- Shift register cleared
- Error flags cleared

## Required Internal Modules

- receiver_fsm
- shift_register
- parity_checker
- baud_counter
- top_module

## Deliverables

- Verilog source code